BAPATLA ENGINEERING COLLEGE: BAPATLA (AUTONOMOUS)

WIRELESS AND MOBILECOMMUNICATION LAB MANUAL

[CODE: 18ECL72]

Prepared by:

Mrs. D. Swetha , M.Tech

Assistant Professor

Mr. Sk.Idrish, M.Tech, (Ph.D)

Assistant Professor

Mr. Y. VENKATA KRISHNA,

LAB ASSISTANT

IV B.Tech.–I Semester

WIRELESS AND MOBILE COMMUNICATIONS LAB

(18ECL72)

| Lectures | 0 | Tutorial | 0 |  | Practical | 3 | Credits | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Continuous Internal Assessment |  |  | : | 50 | Semester End Examination |  |  | : | 50 |

COURSE OUTCOMES (CO):

| Course Outcome (CO) | Course Outcome description |
| --- | --- |
| CO 1 | Understand and analyze fading channel models |
| CO 2 | Understand DSSS modulation technique |
| CO3 | Design and analyze OFDM and MIMO systems |

CO, PO AND PSO MAPPING:

| 18ECL72 | WMC Lab | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO7 | PO8 | PO9 | PO10 | PO11 | PO 12 | PSO 1 | PSO 2 | PSO 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO 1 | Understand and analyze fading channel models | 3 | 2 |  | 2 | 2 |  |  |  |  |  |  |  | 1 | 1 | 3 |
| CO 2 | Understand DSSS modulation technique | 2 | 3 |  | 2 | 2 |  |  |  |  |  |  |  | 2 | 2 | 2 |
| CO 3 | Design and analyze OFDM and MIMO systems | 3 | 2 |  |  | 2 |  |  |  |  |  |  |  | 2 | 2 | 3 |

LIST OF EXPERIMENTS

The following experiments can be performed using Matlab/Simulink/Scilab/Virtual Labs.

- Simulation of Friss Transmission equation.

- Simulation of Rayleigh fading Channel model.

- Calculate the probability that the received signal level crosses a certain sensitivity level.

- Study the outage probability, LCR & ADF in SISO for Selection Combining and MRC.

- Study the effect of handover threshold and margin on SINR and call drop probability and handover probability.

- Study the effect of delay spread on frequency selectivity.

- Plot BER-SNR and Bit Rate-SNR graphs for No Fading  channel

- Plot BER-SNR and Bit Rate-SNR graphs for Flat Fading channel

- Simulation of Okumura Outdoor Propagation Model.

- Simulation of log normal shadowing radio propagation model.

- Simulation of Walsh Hadamard Code.

- Study distribution of downlink C/I due to different parameters.

- Implement Direct Sequence Spread Spectrum modulation technique.

- Design OFDM based Transmitter and Receiver for different channel environments.

- Design OFDM system with 2x2, 2x4, 4x4 MIMO systems.

| Signature of the Lab In-charge (Mr.B.Achyut, Asst.Prof.) |  | Signature of the HOD (Dr. N. Venkateswara Rao, Prof.) |
| --- | --- | --- |

1. Simulation of Friis Free space equation

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clc;

clear all;

close all;

PtdBm=52;

GtdBi=25;

GrdBi=15;

frequency=5*10^9;

lambda=3*10^8/frequency;

d =41935000*(1:1:200);

L=1;

Pt=10^((PtdBm-30)/10);

Gt=10^(GtdBi/10);

Gr=10^(GrdBi/10);

Pr= Pt*(Gt*Gr*lambda^2)./((4*pi.*d).^2*L);

PrdBm=10*log10(Pr)+30;

plot(log10(d),PrdBm);

title('Free space Path loss model')

xlabel('log10(d)');

ylabel('Pr (dBm)');

OUTPUT:-

2. Simulation of RayLeigh fading channel model

Aim:- To write a MATLAB code for simulation of RayLeigh fading channel model

Software:- MATLAB

Code:-

N=1000000;

variance = 0.2;

x = randn(1, N);

y = randn(1, N);

r = sqrt(variance*(x.^2 + y.^2));

step = 0.1; range = 0:step:3;

h = hist(r, range);

approxPDF = h/(step*sum(h));

theoretical = (range/variance).*exp(-range.^2/(2*variance));

plot(range, approxPDF,'b*', range, theoretical,'r');

title('Simulated and Theoretical Rayleigh PDF for variance = 0.5')

legend('Simulated PDF', 'Theoretical PDF')

xlabel('r -?');

ylabel('P®-?');

grid;

theta = atan(y./x);

figure(2)

hist(theta);

[counts,range] = hist(theta,100);

step=range(2)-range(1);

approxPDF = counts/(step*sum(counts));

hold on

plotHandle=plot(range, approxPDF,'r');

set(plotHandle,'LineWidth',3.5);

axis([-2 2 0 max(approxPDF)+0.2])

hold off

title('Simulated PDF of Phase of Rayleigh Distribution ');

xlabel('\theta  --->');

ylabel('P(\theta) --->');

grid;

OUTPUT:-

3. Simulation of Log normal/ Log distance shadowing path loss model

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clear all;

close all;

PtdBm=52;

GtdBi=25;

GrdBi=15;

frequency=1*10^9;

d =41935000*(1:1:500) ;

L=1;

Pt=10^((PtdBm-30)/10);

Gt=10^(GtdBi/10);

Gr=10^(GrdBi/10);

lambda=3*10^8/frequency;

Pr= Pt*(Gt*Gr*lambda^2)./((4*pi.*d).^2*L);

X = sigma*randn(size(Pr));

propLoss = Pr./Pt;

PLdBm = 10*log10(propLoss)+10*log10(X);

PrdBm=10*log10(Pr)+30;

plot(log10(d),10*log10(propLoss),'G','LineWidth',2);

title('Log Normal Shadowing model')

xlabel('log10(d)');

ylabel('P_r/P_t (dB)');

hold on;

plot(log10(d),PLdBm,'r.');

legend('Propagation Loss','Due to Log normal Shadowing');

OUTPUT:-

4. Simulation of Okumura Outdoor Propagation Model

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clc;

clear all;

close all;

Hte=30:1:100;

Hre=input('Enter the receiver antenna height 3m<hre<10m : ');

d =input('Enter distance from base station 1Km<d<100Km : ');

f=input('Enter the frequency 150Mhz<f<1920Mhz : ');

c=3*10^8;

lamda=(c)/(f*10^6);

Lf = 10*log((lamda^2)/((4*pi)^2)*d^2);

Amu = 35;

Garea = 9;

Ghte = 20*log(Hte/200);

if(Hre>3)

Ghre = 20*log(Hre/3);

else

Ghre = 10*log(Hre/3);

end

L50 = Lf+Amu-Ghte-Ghre-Garea;

display('Propagation pathloss is : ');

disp(L50);

plot(Hte,L50,'LineWidth',1.5);

title('Okumura Model Analysis');

xlabel('Transmitter antenna Height (Km)');

ylabel('Propagation Path loss(dB) at 50 Km');

grid on;

OUTPUT:-

Enter the receiver antenna height 3m<hre<10m : 10

Enter distance from base station 1Km<d<100Km : 100

Enter the frequency 150Mhz<f<1920Mhz : 900

Propagation pathloss is :

Columns 1 through 11

59.3736   58.7178   58.0828   57.4674   56.8704   56.2906   55.7272   55.1792   54.6458   54.1263   53.6200

Columns 12 through 22

53.1261   52.6442   52.1736   51.7138   51.2643   50.8247   50.3946   49.9735   49.5612   49.1571   48.7611

Columns 23 through 33

48.3727   47.9917   47.6179   47.2509   46.8905   46.5365   46.1887   45.8468   45.5107   45.1801   44.8549

Columns 34 through 44

44.5349   44.2199   43.9098   43.6045   43.3037   43.0074   42.7154   42.4277   42.1440   41.8642   41.5884

Columns 45 through 55

41.3163   41.0478   40.7829   40.5215   40.2634   40.0086   39.7570   39.5086   39.2632   39.0208   38.7812

Columns 56 through 66

38.5445   38.3106   38.0794   37.8508   37.6248   37.4014   37.1804   36.9618   36.7456   36.5317   36.3200

Columns 67 through 71

36.1106   35.9033   35.6982   35.4952   35.2942

5. Simulation OF WALSH HADAMARD CODE

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clc;

clear all;

N=2;

H=[0 0;0 1];

codeSize=input('enter codeSize value');

if bitand(codeSize,codeSize-1)==0

while(N~=codeSize)

N=N*2;

H=repmat(H,[2,2]);

[m,n]=size(H);

for i=m/2+1:m

for j=n/2+1:n

H(i,j)=~H(i,j);

end

end

end

else

disp('invalid codesize: code must be the power of 2');

end

disp(H);

OUTPUT:-

enter codeSize value8

0     0     0     0     0     0     0     0

0     1     0     1     0     1     0     1

0     0     1     1     0     0     1     1

0     1     1     0     0     1     1     0

0     0     0     0     1     1     1     1

0     1     0     1     1     0     1     0

0     0     1     1     1     1     0     0

0     1     1     0     1     0     0     1

6.  Calculation of probability that the received signal level crosses a certain sensitivity level.

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

Pt=10*10^-3;

f=1*10^9;

c=3*10^8;

lam=c/f;

d0=1;

K=(lam/(4*pi*d0))^2;

d=1.9;

gam=2;

PrFriss=Pt*K*(d0./d).^gam;

gam=4.5;

Pr=Pt*K*(d0/d)^gam;

vari=4

Pmin=10*log10(Pr/4)

x=(Pmin-10*log10(Pt)-10*log10(K)+10*gam*log10(d/d0))/vari

Prob_req=qfunc(x)

OUTPUT:-

>> calofprob6

vari =

4

Pmin =

-71.0063

x =

-1.5051

Prob_req =

0.9339

7. Study the effect of delay spread on frequency selectivity

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clear all

close all

clc

an=[1 0.3 -0.8 0.5 -0.4 0.2];

tn=[0 1 2 3 4 5;0 0.1 0.2 0.3 0.4 0.5]*(10^-6);

dt=0.01*10^-6; t=0:dt:15*10^(-6); t=t(1:end-1);

st=zeros(1,length(t));

st(t<=5*10^-6)=1;

for c=1:2

for i=1:length(an)

del=round(tn(c,i)/dt);

Ray(i,:)=an(i)*[zeros(1,del) st(1:length(st)-del)];

end;

yt(c,:)=sum(Ray);

end

subplot(221)

plot(t*10^6,st)

xlabel('time (\mu sec)');

ylabel('Tx signal');

axis([0 20 -1 2])

grid on

subplot(222)

plot(t*10^6,yt(1,:));

xlabel('time (\mu sec)');

ylabel('Rx signal');

title('Large delay spread')

axis([0 20 -1 2])

grid on

subplot(224)

plot(t*10^6,yt(2,:));

xlabel('time (\mu sec)');

ylabel('Rx signal');

title('Small delay spread')

axis([0 20 -1 2])

grid on

RMS_delay_spread_1=sqrt(var(tn(1,:)))/(10^-6)

display('micro sec');

RMS_delay_spread_2=sqrt(var(tn(2,:)))/(10^-6)

display('micro sec');

OUTPUT:-

8. Plot BER-SNR and Bit Rate-SNR graphs for fading channel of SISO with no fading  (AWGN)

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clear all

close all

clc

SNR=0:1:20;

SNRn=10.^(SNR/10);

N=800; % Number of bits

M=60; % Number of Experiments

fBER=zeros(1,length(SNR));

for mo=1:M

xbase=sign(randn(1,N)); %BPSK

Sigpow=1;

for ss=1:length(SNR)

% Assuming channel is known at the receiver

noipow=Sigpow/SNRn(ss);

% AWGN channel

ybase=xbase+sqrt(noipow)*(randn(1,N));

ydet=(sign((ybase)));

err_pos=find(ydet~=xbase);

BERa(ss)=length(err_pos)/N;

end

fBER=fBER+BERa;

end

% Theoretical Approximate (Analytical) formulae

BERanaAWGN=0.5*exp(-SNRn/2);

%Plotting

semilogy(SNR,fBER/M,'k','LineWidth',3)

hold on

semilogy(SNR,BERanaAWGN,'ko','LineWidth',3);

hold on

grid on

xlabel('SNR (dB)');

ylabel('BER');

legend('AWGN channel (Simulated)','AWGN channel (Theory)');

OUTPUT:-

9. Plot BER-SNR and Bit Rate-SNR graphs for fading channel of SISO with Rayleigh Flat Fading Channel

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

clear all

close all

clc

SNR=0:4:40;

SNRn=10.^(SNR/10);

N=800; %Number of bits

M=100; % Number of Experiments

fBER=zeros(1,length(SNR));

for mo=1:M

xbase=sign(randn(1,N)); %BPSK

cc=0.707*(randn(1,N)+sqrt(-1)*randn(1,N));

Sigpow=1;

for ss=1:length(SNR) %Assuming channel is known at the receiver

noipow=Sigpow/SNRn(ss); % AWGN channel

ybase=xbase.*cc+sqrt(noipow)*(randn(1,N));

yfa=conj(cc).*ybase; ydet=(sign(real(yfa)));

err_pos=find(ydet~=xbase);

BERa(ss)=length(err_pos)/N;

end

fBER=fBER+BERa;

end

% Theoretical Approximate (Analytical) formulae

BERana=1./(2*SNRn); %Plotting

semilogy(SNR,fBER/M,'k','LineWidth',3)

hold on

semilogy(SNR,BERana,'ko','LineWidth',3);

hold on

grid on

xlabel('SNR (dB)');

ylabel('BER');

legend('Flat Fading channel (Simulated)','Flat Fading (Theory)');

OUTPUT:-

10. Implementation of Direct-Sequence Spread Spectrum modulation technique

Aim:- To write a MATLAB code for simulation of Friis free space equation

Software:- MATLAB

Code:-

M_sequence;

D=input('Enter the input data sequence: ');

D(D==0)= -1;

M_seq(M_seq==0)=-1;

td=D;

ld=length(D);

td(1:N)=D(1)*M_seq;

for i=2:ld

td(1,length(td)+1:length(td)+N)=D(i)*M_seq;

end;

Tb=1;

Tpn=Tb/N;

fc=3*N;

fs=150*fc;

ts=1/fs;

k=0;

for i=1:N

if M_seq(i)==1

M_seq1(1,k+1:k+fs/N)=1;

k=length(M_seq1);

else

M_seq1(1,k+1:k+fs/N)=-1;

k=length(M_seq1);

end;

end;

k=0;

for i=1:ld

if D(i)==1

D1(1,k+1:k+fs)=1;

k=length(D1);

else

D1(1,k+1:k+fs)=-1;

k=length(D1);

end;

end;

k=0;

for i=1:ld*N

if td(i)==1

td1(1,k+1:k+fs/N)=1;

k=length(td1);

else

td1(1,k+1:k+fs/N)=-1;

k=length(td1);

end;

end;

t=(0:1:fs*ld-1)*ts;

tm=(0:1:fs-1)*ts;

c=cos(2*pi*fc*t);

BPSK=c.*td1;

figure();

subplot(4,1,1);

plot(t,D1,'b','linewidth',1);

grid on;   %Shows grid lines in the graph

legend('NRZ coded Data signal ');    %Shows graph names given

subplot(4,1,2);

plot(tm,M_seq1,'b','linewidth',1);

grid on;   %Shows grid lines in the graph

legend('NRZ coded M_sequence ');    %Shows graph names given

subplot(4,1,3);

plot(t,td1,'b','linewidth',1);

grid on;   %Shows grid lines in the graph

legend('NRZ coded spread spectrum modulated signal');    %Shows graph names given

subplot(4,1,4);

plot(t,BPSK,'b','linewidth',1);

grid on;   %Shows grid lines in the graph

legend('DSSS_BPSK Modulated Signal');    %Shows graph names given

M_SEQUENCE:

clc;            %Clears the command window

clear all;      %clears all the varibles used in previous program

close all;      %closes all pop up windows like graphs opened in previous window

r=input('enter the length of shiftregister r= '); %reads the shift register length and stores in variable r

N=2^r-1;                                          %calculate PN_sequence length

tp=input('enter the valid tap array :');

in=input('enter the sequence of input data array of length r : ');

if r~=length(in)

fprintf('\n \n error : length of input data array must be equal to the length  of shift registers: \n \n');

else

out=in;

for k=1:N

t_out=out(tp(1));

for i=1:length(tp)-1

t_out=xor(t_out,out(tp(i+1)));

end;

tr1=r;

M_seq(k)=out(r);

for j=1:r-1

out(tr1)=d_ff(out(tr1-1));

tr1=tr1-1;

end;

out(1)=t_out;

if k~=N

if out==in

fprintf('\n \n error : given tap input array is not valid : \n \n');

break;

end;

end;

if k==N

if out==in

fprintf('lenth of the m_sequence N = %d \n',N);

fprintf('M_sequence is : \n');

disp(M_seq);

corre=correlation(M_seq,N);

t=-2*N:1:2*N;

grid on;

plot(t,corre);

grid on;axis([-2*N-1 2*N+1 -1 1.5]);

title('autocorrelation of M-sequence : ');

xlabel('timeperiod');

ylabel('amplitude');

else

fprintf(' \n \n error : given tap input array is not valid :\n \n');

end;

end;

end;

end;

OUTPUT:

enter the length of shiftregister r= 3

enter the valid tap array :[3 1]

enter the sequence of input data array of length r : [1 0 0]

lenth of the m_sequence N = 7

M_sequence is :

0     0     1     1     1     0     1

Enter the input data sequence: [1001010]